12. DOOR AND WINDOW INSTALLATION  
Detailed Step-by-Step Door and Window Installation Instructions
Pre-Installation Preparation

Tools Required:

· Hammer

· Ladder/step stool

· Hand saw

· Silicone sealant and application gun

· Screwdriver

· Level

Tools that will be useful or necessary:

[image: image1.emf][image: image3.jpg]


[image: image4.jpg]


[image: image5.jpg]


[image: image6.jpg]


[image: image7.jpg]



[image: image8.png]1. unscrew internal

2. Apply silicone on the
window / doors edgings

external edgings inner
side





Important Timing:

· Wait until at least 5 logs are installed before mounting doors for easier access and structural stability

Important Warnings

⚠️ CRITICAL: Never attach frames directly to walls with nails or screws 
⚠️ CRITICAL: Never fill expansion gaps with solid materials 
⚠️ CRITICAL: Ensure proper orientation before final installation

Quality Control Checklist

Before completing installation, verify:

· Correct window/door orientation

· All silicone properly applied

· Expansion gaps maintained

· Drip caps installed (if applicable)

· No direct attachment to walls

· Frames properly centered

· All edgings securely fastened

Step 1: Prepare for Water Protection

1.1 Silicone Application for Windows

1. Unscrew internal window/door edgings from the frame

2. Apply silicone sealant to the inner side of external edgings

3. Add extra silicone on top of the frame to prevent water infiltration

4. Allow silicone to set according to manufacturer's instructions

1.2 Door Panel Weatherproofing

1. Apply silicone to all edges of the door's solid panel where tongue-and-groove boards meet

2. This prevents rainwater from running down the T&G boards and penetrating joints[image: image9.emf]            


[image: image10.emf]
Window inside/outside view – 1. Make sure that you are fixing the right side of window; 2. Inside view; 3. Outside view; 4. Hinges; 5. Handle; 6. Locker; 7. Hinge clipper
Step 2: Verify Correct Orientation

2.1 Window Orientation Check

· Regular windows: Handle should be on the inside

· "EV" windows: Handle will be on the outside (opposite of regular windows)

· Doors: Use the provided floor plan to determine correct orientation

2.2 Identify Components

From inside view, locate:

· Hinges (left side)

· Handle (bottom center)

· Locker mechanism

· Hinge clipper
Window inside/outside view – 1. Make sure that you are fixing the right side of window; 2. Inside view; 3. Outside view; 4. Hinges; 5. Handle.
Step 3: Install Windows and Doors

3.1 Positioning

1. Place door/window in their designated openings
2. Center the frame to ensure even expansion/contraction gaps on all sides (except bottom)

3. Verify gaps are present - these should already be cut in the wall structure

3.2 Securing

1. Screw back the inner edgings to secure the window/door in place

2. DO NOT nail or screw frames directly to walls - this prevents damage from wood expansion/contraction
Step 4: Critical Gap Management

4.1 Expansion/Contraction Gaps

· Maintain gaps between frame and walls on top and sides (not bottom)

· Never fill gaps with solid materials - this can cause: 

· Wall gapping

· Splitting

· Cracking

· Gaps will be covered by the door/window frames themselves
Step 5: Install Drip Caps (Recommended)

5.1 Drip Cap Installation

1. Position drip caps 50mm higher than door and window frame tops

2. This accounts for potential log settling over time

3. Apply silicone between drip caps and wall where they join

4. Cut angles yourself if ordering drip caps from supplier

5.2 DIY Drip Cap Option

· Use leftover cabin parts to create your own drip caps

· Ensure proper angling for water runoff
Step 6: Final Weatherproofing

6.1 Complete Silicone Sealing

Apply silicone to all red-marked areas including:

· All frame edges

· Between drip caps and wall

· Any remaining joints that could allow water penetration

6.2 Ongoing Maintenance

· Monitor silicone condition regularly

· Renew silicone as needed since it loses properties over time

· Check for any gaps or cracks that develop

   [image: image2.png]red marked spots
also between drip
caps and wall




Protecting door/window –

1. Apply silicone to the red-marked spots, including between the drip caps and the wall; 

2. Drip caps







Window installation – 1. Unscrew internal window / door edgings; 2. Apply silicone on the external edgings inner side; 3. Extra silicone on top to prevent water




















