6. FIRST LOGS OF YOUR CABIN

Tools that will be useful or necessary:

[image: image1.emf][image: image2.jpg]HEIGHT

GROOVE




[image: image3.jpg]


[image: image4.jpg]



ALL LOGS HAVE TO 

[image: image5.jpg]


[image: image6.jpg]



ALL LOGS HAVE TO 
BE PLACED 
TONGUE UP
First logs placement 2 – 1. External base beams; 2. Internal base beams; 3. 65.5mm logs; 4. 131mm logs; 5. Water drip corner (~10mm)
First Log Installation Overview

Log Types:
· 65.5mm width logs - Must be placed first

· 131mm width logs - Placed after the 65.5mm logs are installed

Critical Requirements:
· All logs must be placed tongue up
· A waterproof layer is required between the foundation and first logs

· Proper placement is crucial as "a good start is the key to success"
[image: image11.emf]
Water drip corner
Installation Steps:

1. Placement Order: Install all first logs in the correct sequence, starting with 65.5mm logs

2. Positioning: Center the logs on the foundation with about 10mm overhang for water drip edge

3. Alignment Check: Verify logs are straight using the same thread technique used for base beams

4. Securing: Screw logs in place with approximately 0.5m gaps between screws

Foundation Details:

· External base beams (70x43mm)

· Internal base beams (43x43mm)

· 10mm water drip corner overhang to prevent water damage

[image: image7.jpg]




First logs placement – 1. 65.5mm wall logs (must be placed first); 2. 131mm wall logs (placed after the 65.5mm logs are installed).





First logs placement 2 – 1. External base beams; 2. Internal base beams;   3. 65.5mm logs; 4. 131mm logs; 5. Water drip corner (~10mm)








3

[image: image8.jpg]


[image: image9.png]


[image: image10.jpg]


