## 4. FOUNDATION

### Foundation Problems to Avoid

- Strong Base ✅ - Level, stable foundation

- Soft Base ❌ - Uneven, unstable ground causing cabin tilt

- Water under Base ❌ - Poor drainage causes structural problems

- Empty Pockets ❌ - Gaps create instability and settling

### Foundation Benefits

A properly prepared foundation ensures:

- Easy log wall assembly - Components fit together smoothly

- Proper door and window operation - No binding or gaps

- Long-term structural integrity - Prevents settling and warping

- Moisture protection - Keeps cabin dry and prevents rot

### Before You Begin

Remember: Foundation materials are not included with your log cabin kit. You must source and prepare the foundation separately before assembly begins.

Pro Tip: Take extra time with foundation preparation. An even, stable foundation is the key to successful cabin assembly and years of trouble-free use.

### 🏗️ Critical Foundation Standards

Foundation must be:

- Level and flat: Essential for complete stability

- Well-draining: Ground must permeate water well

- Frost-protected: Prevent frost heaving damage

- Load-bearing: Capable of supporting the full weight of the structure

- Moisture-proof: No wooden components should contact moisture or soil

### 🏗️ Recommended Foundation Options

1. Concrete Slab Foundation (Recommended)

- Most stable and lasting option

- Build at least 25cm above ground level

- Provides excellent moisture protection

- Best long-term investment

2. Concrete Sole Foundation

- Alternative to full slab

- Must be 25cm above ground level

- Good stability with lower cost

3. Wooden Foundation

- Natural appearance option

- Requires proper moisture separation

- Foundation materials NOT included in package

### 💧 Critical Moisture Protection

Wooden parts are not close friends with humidity, even if impregnated:

- Always separate wooden base beams from concrete with waterproof layer

- Use any waterproof material of your choice

- This prevents moisture damage and extends cabin life