# 7. Half Log Connection Assembly Guide

Most log lock connections come pre-cut and ready to assemble. In most cases, you'll simply need to put the pieces together and secure them with screws.

## Standard Assembly Process

### Step 1: Identify Connection Points

Look for the pre-cut connection points on your log pieces. These connection areas are typically marked or easily identifiable on the logs.

### Step 2: Align and Join

- Position the log pieces so the connections align properly

- Fit the pieces together at the connection points

- Ensure a snug, secure fit

### Step 3: Secure with Screws

- Insert screws through the designated holes

- Tighten screws to secure the connection

- Check that the joint is stable and properly aligned

## Special Case: Uncut 65.5mm Logs

In rare instances, 65.5mm height logs may arrive with uncut log joints that require manual preparation.

65.5 mm height wall log connection – 1. Cutting line (must be in the middle of the log lock); 2. Join both cut parts; 3. The final log lock size must match the thickness of your cabin wall (44-70 mm) – please always check and cut these marked parts if required.

### Cutting Instructions

Step 1: Mark the Cutting Line

- Locate the center of the log lock

- Mark a cutting line exactly in the middle of the log lock

Step 2: Make the Cut

- Cut along the marked line using appropriate tools

- Ensure the cut is straight and clean

Step 3: Join the Cut Parts

- Align the two cut sections

- Test fit the pieces together

### Critical Measurement Check

Final Size Verification:

- The completed log lock must match your cabin wall thickness

- Acceptable range: 44-70mm

- Always measure and verify before final assembly

- Trim if necessary to achieve proper fit

## Quality Control

- Double-check all connections before proceeding

- Ensure all joints are secure and properly aligned

- Verify that log lock dimensions match your cabin specifications

## Tools You May Need

- Measuring tape or ruler

- Saw (if cutting is required)

- Screwdriver or drill

- Screws (as specified in your kit)

## Safety Notes

- Always wear appropriate safety equipment when cutting

- Take accurate measurements to avoid mistakes

- Test fit pieces before final assembly