18. The Connection of Top Logs 
(Simple Joint Type)
Overview

Some cabin models feature top logs with a simple joint connection system. This method uses pre-marked joint lines on the logs that need to be aligned and connected properly.

What You'll Need

· Adhesive: Silicone sealant OR wood glue

· Tools: Application gun for silicone (if using) or brush for wood glue

· Reference: Your cabin's axis drawing/assembly diagram

Step-by-Step Instructions

1. Identify Your Log Type

· Check if your cabin model has top logs with simple joint markings

· Look for visible lines on the top logs - these indicate the joint locations

· Refer to your specific cabin's axis drawing for the correct configuration

2. Locate the Marked Parts

· Find all top log pieces that need to be connected

· Identify the joint lines on each log piece

· Match the corresponding pieces according to your assembly diagram (typically labeled as sections A and B)

3. Prepare the Joint Areas

· Clean the joint surfaces to remove any dust or debris

· Ensure the joint lines align properly when pieces are placed together

· Test fit the pieces before applying adhesive

4. Apply Adhesive

Choose ONE method:
Option A - Silicone:
· Apply a continuous bead of silicone along the joint line

· Use enough to ensure good coverage but avoid excess that will squeeze out

Option B - Wood Glue:
· Apply wood glue evenly along the joint surface

· Spread with a brush or applicator for uniform coverage
5. Connect the Logs

· Carefully position the marked parts next to each other

· Align the joint lines exactly as shown in your axis drawing

· Press firmly to ensure good contact and adhesion

· Remove any excess adhesive that squeezes out immediately

6. Secure the Connection

· Hold the pieces in position until the adhesive begins to set

· Follow the adhesive manufacturer's recommended curing time

· Avoid moving or stressing the joint during the curing period

Important Notes

· Always reference your specific cabin's axis drawing for exact positioning

· The connection method creates a strong, permanent joint

· Work in suitable weather conditions if using outdoors

· Allow proper curing time before proceeding with additional assembly steps

Troubleshooting

· If joint lines don't align: Double-check you have the correct matching pieces

· If gaps appear: Ensure surfaces are clean and flat before reapplying adhesive

· For questions about your specific model: Consult your complete assembly manual

[image: image1.jpg]Connection of two top logs

L&

T

Bl



This simple joint system provides an effective way to create strong, seamless connections between top log sections in your cabin construction.

[image: image2.emf]
Simple wall logs connection type – 1. Use silicone or wood glue 








